JEFF LANDRY

GOVERNOR

State of Louisiana

Statewide Interoperability Executive Committee

https://gohsep.la.gov/about/unified-command-group/interoperability-subcommittee/

MARK WARD

CHAIRMAN

# POLICY NAME: NETWORK PATCHING POLICY

# POLICY NUMBER 006

# EFFECTIVE DATE: January 27, 2010

# REVISED DATE: July 22, 2015; July 27, 2016, July 24, 2019, January 21, 2026

LWIN INTEROPERABILITY PATCHING POLICY

# PURPOSE

The purpose of this policy is to define a Louisiana Wireless Information Network (LWIN) Interoperability Patch and the operation of a patch.

Due to the existence of disparate systems in the State of Louisiana, the Statewide Interoperability Executive Committee (SIEC) has determined the need for a policy concerning patching these systems to LWIN. Generally, the patching of LWIN is discouraged during normal operation due to the high risk of significantly degrading availability of system resources. The SIEC encourages agencies to discontinue patches as soon as reasonably possible.

The implementation of the patch hardware, interface, radio, labor, etc. is the responsibility of the requesting member agency. The state may invoice the member agency for any technical support supplied by state personnel.

Examples of patches that may need to be made are:

  - Cross patch to municipal, parish, or regional trunked systems

  - Cross patch to conventional system

  - Talk group patch within LWIN

  - Patch to a separate frequency band

  - Patch to cell phones/satellite phone

# DEFINITION

An LWIN Patch is defined as the interconnection of the LWIN system to a private

branch exchange (PBX) or other telephone system, cell systems, the internet, satellite phones or another agency’s communication system.

In most cases LWIN patches can be accomplished through dispatch consoles or external devices such as an ACU1000, RIOS, Motobridge, a conventional channel gateway (CCGW), with CCGW being the preferred method.

Three types of approved LWIN Patches are identified in this policy:

- Temporarily Patch.

  - A patch that is set up for a specific event, designed to be disconnected at the conclusion of that event.

    - Example 1: Patching a channel from an agency responding from out of state to an LWIN talkgroup to allow interoperable communications.

    - Example 2: A vehicle pursuit crossing jurisdictional lines requiring talkgroups from different law enforcement agencies on LWIN to be patched to allow seamless interoperable communication to the end users.

- Permanent Patch.

  - Patch set up without regard to an event, designed to remain in place with no time frame for disconnecting.

    - Example: Fire Departments use of patching to extend the range of Station Alerting Systems to units in the field by patching them to a specific talkgroup on the LWIN system.

    - Note: permanent patches are designed to link LWIN with other communications resources, not to patch LWIN talkgroups. If there is a need for agencies to share a talkgroup indefinitely a single talkgroup will be created or assigned.

- ROIP Patch.

  - The preferred method for connecting a ROIP solution will be a wireline connection to LWIN in coordination with LSP Mobility Communications Services (MCS).

For the purpose of this policy, statewide interoperability talkgroups/channels are STATE 1-4, INTEROP 1-12, LE SECURE INTEROP 1-16 and Gulf Coast Interoperability (GCI) 1-16.

# PROCEDURES AND GUIDELINES

- The following guidelines shall be used for an LWIN patch deployment:

# Temporary Patches

- LWIN member agencies will make emergency temporary patches as they identify a need with no prior approval required.

- Agencies will notify the Louisiana State Police (LSP) MCS section of any temporary patch enabled that remains in place for more than twelve hours.

Contact Information for LSP MCS is provided on the SIEC/LWIN Website:

https://gohsep.la.gov/about/unified-command-group/interoperability-subcommittee/

- If a patched talkgroup is proprietary to a specific jurisdiction, the authority of that jurisdiction must sign a Memorandum of Understanding (MOU) granting use of the proprietary talkgroup(s). In the case of quickly developing situations, verbal authorization may be substituted until an MOU can be created.

- The patch must function in a technically and operationally consistent manner.

  - The release time between messages should be less than 4 seconds.

  - The audio quality should be a close representation of the original audio as heard on a typical subscriber radio.

  - The audio shall be free of hum, clicks, or other extraneous noise.

  - There shall be no clipping of the first syllables or loss of audio through the patch.

- Maintenance of the temporary patch and facilities is the responsibility of the member agency initiating the patch.

- The agency shall continuously monitor and respond to calls on the patch.

- Although necessary, patches may degrade the performance of the LWIN system; as such, agencies are required to disconnect temporary patches as soon as possible.

# Permanent Patches

- Permanent patches will be approved by the SIEC after a need has been demonstrated. LWIN Grade of Service will be considered in the approval of a permanent patch.

Permanent patches are to remain active at all times on the Talkgroup specified within the Patch Agreement [Appendix A]. This requirement is to provide the public safety users with a consistent and functioning communications path between the radio systems.

- When utilizing external patching devices or bridging equipment such as an ACU 1000, only one talkgroup will be programmed into a permanently patched LWIN interface radio to ensure the radio remains on the patched talkgroup.

- The patch must function in a technically and operationally consistent manner.

  - The release time between messages should be less than 4 seconds.

  - The audio quality should be a close representation of the original audio as heard on a typical subscriber radio.

  - The audio shall be free of hum, clicks, or other extraneous noise.

  - There shall be no clipping of the first syllables or loss of audio through the patch.

- LWIN Statewide talkgroups cannot be permanently patched.

- Maintenance of a permanent patch is the responsibility of the member agency initiating the patch.

# LWIN PATCH REQUEST

- Agency to LWIN

When an agency needs to perform a permanent patch or a temporary patch lasting more than twelve hours to LWIN that will be accomplished with no outside assistance, the agency must provide the following information to ESF-2:

  - Agency requiring network patch

  - Reason for request or event type, i.e. hurricane, floods, fire etc.

  - Details of the patch to include type of system, frequency or talkgroup

  - All involved agencies requesting interoperability

  - Expected duration of event

  - Patch Location

  - A point of contact including phone number

# Agency to LWIN requiring assistance

Agencies may request use of the resources from ESF-2 by providing the following information:

  - Agency requesting network patch

  - Reason for request or event type, i.e. hurricane, floods, fire etc.

  - Details of the patch to include type of system, frequency or talk group

  - All involved agencies requesting interoperability

  - Equipment required

  - Expected duration of event

  - Location required/access information

  - A point of contact including phone number

# TACTICAL PATCH ACTIVATION AND USE

If a tactical patch (initiated in the field and operated from a Tactical Command Center) is needed for a particular quickly developing event ALL agencies involved that are not able to access common mutual aid/Interoperability channels/talkgroups will be required to bring a portable radio to the tactical equipment command center (TECC) for the length of the operation. Setup and installation of all radios will occur at location of patch. Agencies are also responsible for providing additional power supplies (i.e., spare batteries, chargers, speaker microphones, necessary cables, etc.) for portable radios, as battery life limits usability of the radio (see Network Patch Limitations below).

The Incident Commander or designee should follow these procedures in accordance with the National Incident Management System (NIMS):

  - Avoid using an agency's primary dispatch channel

  - Require participating agencies to check in at the command post and provide portable radios and frequency/talk group channels for use during the incident to the Communication Unit Leader (COML) and/or Communications Technician (COMT).

  - Assign radio call sign/designator information to connected agencies.

  - Instruct TECC on where to set up and operate the tactical equipment if assigned

  - Inform ESF-2 and TECC which agencies are participating

  - Provide TECC with agency provided radios and an Incident Command Structure (ICS) form 205 with the frequency/talk group channels to be used during the incident

  - Confer with ESF-2 and TECC concerning what command level or other specific talk groups that need to be programmed into the network patch

TECC should follow these procedures:

  - Arrange to obtain agency radios and connect to the patch with associated cables

  - Select the channel or talk group assigned by the agency

  - Assign the requested unit/agency to that channel or talk group as designated by the Incident Commander

  - Coordinate and/deconflict Communications Plan (205 with ESF-2 & MCS.

# PATCH DEACTIVATION

When interoperable communications patches are no longer required, agencies should follow these guidelines.

The Incident Commander or designee:

  - Make an announcement on the incident command channel to all commanders to advise them the network patch is being deactivated.

    - Contact the TECC and/or console operator to shut down the patch.

Participating Agencies:

- Individual agencies are responsible for retrieving the portable radio and    associated equipment provided during the operation.

TECC:

  - Assure agencies retrieve all portable radios

  - Take inventory of equipment and note any needing repair or replacement

  - Return to pre-response storage location and make the tactical equipment ready for service

  - Contact ESF-2 and MCS to advise of the disconnection of the patch.

NOTE: This procedure (VI) should be followed when disconnecting any patched channels/talkgroups.

# PROBLEM ID AND RESOLUTION

If an issue or problem is identified during the ANY patch, ESF-2 and/or MCS shall determine who will take corrective action. If the issue or problem cannot be identified, ESF-2 or MCS shall contact the appropriate technical personnel or Parish Emergency Operation Center (EOC) to address the issue or problem.

Any problems discovered during the patch shall be resolved in the following manner:

  - The local dispatch center having jurisdiction over the location of the incident reports any problems experienced to the Network Control Center (NCC)

  - The Incident Commander or designee will be responsible for ensuring effective resolution of problems that exist with interoperability resources and notify the local dispatch center of the issues' resolution.

The following guidelines shall govern patch problem identification and resolution between agencies:

  - ESF-2 and/or MCS reports any problems experienced to the Incident Commander or designee. Agencies using network patches may also report any problems experienced.

  - The Incident Commander or designee will be responsible for ensuring effective resolution of problems that exist with interoperability resources and notify the local dispatch center of the issues' resolution.

  - If a patch is detected and is found to be adversely affecting the performance of the LWIN, and the means to contact the Incident Commander or designee fails, the ESF-2 reserves the right to remotely “inhibit” or turn off the radio that is being used to patch to other systems. This means shall be used as a last resort to preserve the LWIN system performance for all other users.

# LWIN PATCH TEST PROCEDURES

To ensure that equipment components of the network patch operate properly, each agency will test their resources according to their agency's individual policies and procedures. Below are recommended procedures:

  - Representatives from each agency should meet on a regular basis to test communications

  - Testing should include deployment, setup, operation, and deactivation of the network patch

Agency representatives should arrive at the test location to test their ability to communicate with other agencies utilizing the patch.

# INTERSTATE PATCH TO AND FROM LWIN

  - Any and all cross-state border patches will be initiated by and at the direction and control of the ESF-2 leads and the NCC.  No other entity has the authority to initiate any such connections to or from the LWIN system.  Any requests for connections of this type must be sent, in writing to: https://gohsep.la.gov/about/unified-command-group/interoperability-subcommittee/